Spanning Tree Protocol (STP)

- Layer 2 protocol

Problem: Routing Loop In Layer 3

- Dynamic Routing protocols have built-in loop prevention

- TTL acts as a final failsafe

Ethernet Path Selection

- Without STP

  - Layer 2 loops

  - Ethernet does not have TTL

  - Broadcast Storm

  - Must be avoided at all costs

- With STP

  - We block some ports for preventing the Broadcast Storm

  - It’s the necessary evil

  - There are problems with the bandwidth but it’s necessary sometimes

STP Terminology

- The BRIDGE

  - Layer 2 Device

  - Learns MAC addresses from the 2 sides

  - They are early switches

  - A switch is a multi-port bridge

  - STP was invented back when bridges were in use (‘The root bridge’)

How Spanning Tree Works?

- Enabled by default on all vendors’ switches

- Switches send Bridge Protocol Data Units when they come online

- BDPU contsins the switch’s Bridge ID

- Bridge priority 0-65535, with 32768 default

- Lower Bridge Priority is preferred

- If a tie, lowest MAC address will be selected

- Short-Mode vs Long-Mode

- Root is the best BRIGdE id

- Lowest cost paths are preferred (in funcite de viteza interfetelor)

- dupa calcularea rutelor, se gasesc path-uri care nu se folosesc si se blocheaza

- the Root bridge is elected and then the other switches chooses one Root port for arriving to the root bridge

- designated ports – the facing ports of the root ports

- blocked port

- BDPU will continue to be sent but

Spanning Tree Versions

Cisco Versions

Spanning Tree Ports Roles

The Root Bridge Election

- Default value priority 32768

- Lowest MAC Address is the one chosen

Spanning Tree and HSRP Relationship

Spanning Tree Portfast

- when port becomes active, it takes Spanning Tree 30 seconds to ensure that it’s not a loop

- on brigdge port it broadcasts on all ports

- for not waiting for 30 seconds (if you are sure that port is not gonna create loop) PORTFAST should be enabled

- Postfast Ports are typically access ports to end hosts

- Trunk ports should not be Postfast Ports

- to prevent any loops for Postfast Ports UDPU GUARD is used

- if a port is shut by UDPU GUARD, then ensure it’s shutdown and no shutdown after

Spanning Tree Root Guard

- in cazul in care se introduce un switch in network, acesta s-ar putea sa fie noul root bridge, dar nu vrem asta

- de aceea se folosesti Spanning Tree root guard

pt cele 2 pc-uri pot active PortFast

Spanning tree BDPU Filter

- switches send BDPUs, end hosts do not

- works differently depending on global or interface level config

COMMANDS

#show spanning-tree pathcost method

#spanning-tree pathcost method long

#show spanning-tree vlan 1

#show mac address-table

Seteaza un switch ca fiind root sau secundar

#spanning-tree vlan 1 root primary

#spanning-tree vlan 1 root secondary

#spanning-tree vlan <id> priority (priority:0-61440, increment of 4096)

Portfast

BDPU Guard

Spanning Tree Root Guard

Spanning Tree Root Guard

Spanning Tree BDPU Filter