Spanning Tree Protocol (STP)

- Layer 2 protocol

Problem: Routing Loop In Layer 3

- Dynamic Routing protocols have built-in loop prevention

- TTL acts as a final failsafe

Ethernet Path Selection

- Without STP

  - Layer 2 loops

  - Ethernet does not have TTL

  - Broadcast Storm

  - Must be avoided at all costs

- With STP

  - We block some ports for preventing the Broadcast Storm

  - It’s the necessary evil

  - There are problems with the bandwidth but it’s necessary sometimes

STP Terminology

- The BRIDGE

  - Layer 2 Device

  - Learns MAC addresses from the 2 sides

  - They are early switches

  - A switch is a multi-port bridge

  - STP was invented back when bridges were in use (‘The root bridge’)

How Spanning Tree Works?

- Enabled by default on all vendors’ switches

- Switches send Bridge Protocol Data Units when they come online

- BDPU contsins the switch’s Bridge ID

- Bridge priority 0-65535, with 32768 default

- Lower Bridge Priority is preferred

- If a tie, lowest MAC address will be selected

- Short-Mode vs Long-Mode

- Root is the best BRIGdE id

- Lowest cost paths are preferred (in funcite de viteza interfetelor)

- dupa calcularea rutelor, se gasesc path-uri care nu se folosesc si se blocheaza

- the Root bridge is elected and then the other switches chooses one Root port for arriving to the root bridge

- designated ports – the facing ports of the root ports

- blocked port

- BDPU will continue to be sent but

Spanning Tree Versions

Cisco Versions

Spanning Tree Ports Roles

The Root Bridge Election

- Default value priority 32768

- Lowest MAC Address is the one chosen

COMMANDS

#show spanning-tree pathcost method

#spanning-tree pathcost method long

#show spanning-tree vlan 1

#show mac address-table

Seteaza un switch ca fiind root sau secundar

#spanning-tree vlan 1 root primary

#spanning-tree vlan 1 root secondary

#spanning-tree vlan <id> priority (priority:0-61440, increment of 4096)